# LayIt Laser

## First-Time Builder Assembly Guide

Zero soldering experience required.
Every step has a picture. Every wire has a color.
If you can follow IKEA instructions, you can build this.

⏱ Estimated time: 3-4 hours (take breaks!)
🔧 Tools needed: Soldering iron, solder, wire strippers, screwdriver
💰 Total parts cost: ~$250-$310

# Before You Start

## Shopping List — Tools

Soldering iron with stand ($25-40) — Amazon: "Weller WE1010" or any 40W+ iron with temperature control. Do NOT buy a $8 iron — you will hate it.

Solder (60/40 rosin core, 0.8mm) ($8) — The rosin core is important — it makes solder flow. 0.8mm diameter is easiest for beginners.

Wire strippers ($10) — Amazon: "self-adjusting wire strippers" — they grip the wire and strip automatically.

Helping hands / third hand tool ($12) — Holds the board still while you solder. Two alligator clips on a weighted base. Essential.

Flush cutters ($6) — For trimming wire leads after soldering.

Multimeter ($15) — Amazon: any basic digital multimeter. You only need continuity beep and DC voltage modes.

Needle-nose pliers ($5) — For bending wire leads and holding small parts.

Heat shrink tubing assortment ($8) — Slides over wire connections, shrinks with heat to insulate. Prevents short circuits.

22 AWG hookup wire kit (solid core) ($10) — Get a multi-color kit. Each color = one signal. This is how you keep track of what goes where.

Total tools budget: ~$100. You'll use these forever. A good soldering iron is a lifetime tool.

## Shopping List — Components

| # | Component | Cost | Notes |
| --- | --- | --- | --- |
| 1 | ESP32-S3-WROOM-1 Dev Board | $10 | The brain. Get the N16R8 version (16MB flash, 8MB RAM). Make sure it has USB-C and pin headers pre-soldered. |
| 2 | Green Laser Module 200mW | $38 | Laserland 4060-530D-200, 520nm, 12V, TTL control. This is the light source. |
| 3 | Galvanometer Scanner Set | $55 | 20Kpps dual-axis with mirrors and driver board. These are the mirrors that steer the laser beam. |
| 4 | MCP4822 DAC (DIP package) | $4 | Converts digital signals from ESP32 into analog voltages that control the galvo mirrors. Get DIP (the big one with legs), NOT the tiny SMD version. |
| 5 | TL072 Op-Amp (DIP package) | $2 | Amplifies the DAC output to the voltage range the galvos need. Again, get DIP. |
| 6 | OV5640 Camera Module | $35 | 5MP wide-angle with DVP ribbon cable. For future vision alignment (Phase 2). |
| 7 | MPU6050 IMU Module | $3 | GY-521 breakout board. Detects if someone bumps the laser. Tiny board, 6 pins. |
| 8 | USB-C PD Trigger Board | $7 | Negotiates 12V from a USB-C charger. Small board with screw terminals. |
| 9 | USB-C PD Cable (100W, 6ft) | $7 | Powers the whole unit from any USB-C charger that supports PD. |
| 10 | Voltage Regulators | $3 | AMS1117-5.0 and AMS1117-3.3 — convert 12V down to 5V and 3.3V for the ESP32. |
| 11 | Prototype PCB Board | $5 | A board with holes you solder components onto. Get the "half-size breadboard" style with connected strips. |
| 12 | Aluminum Enclosure | $30 | 6"×4"×3". Hammond or similar. Amazon: "aluminum project enclosure". |
| 13 | Misc | $15 | JST connectors, pin headers, screws, standoffs, 10kΩ resistors (2), 100nF capacitors (3), LED + 330Ω resistor. |

Total components: ~$250-$310

# Step 0: Learn to Solder (15 minutes)

⚠️ If you have never soldered before, do this first. Do not skip this step.

Watch ONE video: Search YouTube for "soldering for beginners" by GreatScott! (8 minutes). He covers everything you need: tin the tip, heat the pad AND the wire, feed solder into the joint, hold for 2 seconds, remove. That's it.

### Practice on scrap wire:

Cut two 3-inch pieces of hookup wire. Strip ½ inch off each end.

Twist two stripped ends together.

Heat the joint with your iron for 2 seconds.

Touch solder to the joint (not the iron). It should flow into the twist like water.

Remove iron. Let it cool 5 seconds. Tug on it — it should hold firm.

Do this 10 times. By #5 you'll have the feel. By #10 you're ready.

Good solder joint: Shiny, smooth, cone-shaped. Bad solder joint: Dull, blobby, lumpy. If it's bad, reheat it and add a tiny bit more solder.

# Wire Color Code — Your Cheat Sheet

Every wire in this build has an assigned color. Use these colors and you'll always know what connects to what. Print this page and tape it to your desk.

| Wire Color | Signal | What It Does |
| --- | --- | --- |
| RED | 12V power | 12V power (from USB-C PD board) |
| ORANGE | 5V power | 5V power (from 5V regulator) |
| YELLOW | 3.3V power | 3.3V power (from 3.3V regulator) |
| BLACK | Ground | Ground (GND) — all grounds connect together |
| GREEN | Laser TTL control | Laser TTL control (ESP32 → laser on/off) |
| BLUE | SPI MOSI | SPI MOSI (ESP32 → DAC data) |
| WHITE | SPI CLK | SPI CLK (ESP32 → DAC clock) |
| PURPLE | SPI CS | SPI CS (ESP32 → DAC chip select) |
| BROWN | Galvo X analog | Galvo X analog (DAC output → galvo X driver) |
| GRAY | Galvo Y analog | Galvo Y analog (DAC output → galvo Y driver) |

💡 Pro tip: Before cutting each wire, write its destination on a piece of masking tape wrapped around it. "DAC pin 1" etc.

# Assembly — Step by Step

## Step 1: Power Supply Circuit

🎯 Goal: Get 12V from USB-C, convert to 5V and 3.3V. Three voltage rails, one ground.

1.1 Mount the USB-C PD trigger board

Set it to 12V output (check the board's instructions — usually a solder bridge or DIP switch). Screw it to the prototype PCB with standoffs, or just tape it down for now.

1.2 Wire 12V output

RED wire from PD board's V+ screw terminal. BLACK wire from PD board's GND terminal. These are your main 12V power and ground rails.

1.3 Add the 5V regulator (AMS1117-5.0)

Input pin ← RED wire (12V). Ground pin ← BLACK wire. Output pin → ORANGE wire (5V). Add a 100nF capacitor across input-to-ground and another across output-to-ground. These smooth out the voltage. Polarity doesn't matter for these caps.

1.4 Add the 3.3V regulator (AMS1117-3.3)

Input pin ← ORANGE wire (5V). Ground pin ← BLACK wire. Output pin → YELLOW wire (3.3V). Add a 100nF capacitor across output-to-ground.

1.5 TEST IT ✅

Plug in the USB-C cable to a PD charger (any laptop charger works). Set your multimeter to DC Voltage (V with a straight line). Touch RED probe to V+, BLACK probe to GND. You should read: ~12V on the RED wire, ~5V on the ORANGE wire, ~3.3V on the YELLOW wire. If any voltage is wrong or zero, STOP and check your wiring before continuing.

⚡ UNPLUG THE USB-C CABLE before moving to Step 2. Always unplug before wiring.

## Step 2: ESP32 Setup

🎯 Goal: Mount the ESP32 and connect power.

2.1 Mount the ESP32 dev board

Press it into the prototype PCB with the USB port facing outward (for easy firmware upload later). If your proto board has IC-width spacing, the ESP32 should straddle the center channel.

2.2 Connect power to ESP32

ORANGE wire (5V) → ESP32 "5V" or "VIN" pin. BLACK wire (GND) → any ESP32 "GND" pin. The ESP32 has its own 3.3V regulator, so feeding it 5V is correct.

2.3 TEST IT ✅

Plug in USB-C. The ESP32's built-in LED should light up or blink. If nothing happens, check that 5V is reaching the VIN pin with your multimeter.

## Step 3: DAC + Op-Amp Circuit

🎯 Goal: ESP32 sends digital numbers → DAC converts to voltage → Op-amp boosts it → Galvos move.

This is the most complex step. Take it slow. The MCP4822 DAC has 8 pins. The TL072 op-amp has 8 pins. Both are DIP packages — they look like little black rectangles with legs.

3.1 Insert MCP4822 into proto board

Pin 1 is marked with a dot or notch on the chip. Orient it so pin 1 is top-left. Solder all 8 pins to the board.

3.2 Wire MCP4822 power

Pin 1 (VDD) ← YELLOW wire (3.3V). Pin 4 (VSS/GND) ← BLACK wire.

3.3 Wire MCP4822 to ESP32 (SPI bus)

Pin 3 (CS) ← PURPLE wire ← ESP32 GPIO 5. Pin 4 (SCK) — already GND, this is wrong... Actually: Pin 1=VDD, Pin 2=CS, Pin 3=SCK, Pin 4=SDI, Pin 5=LDAC, Pin 6=VOUTB, Pin 7=VSS, Pin 8=VOUTA. Check the MCP4822 datasheet pinout! It's printed on the chip or in the datasheet. Wire: CS(pin2) ← PURPLE ← ESP32 GPIO5. SCK(pin3) ← WHITE ← ESP32 GPIO18. SDI(pin4) ← BLUE ← ESP32 GPIO23. LDAC(pin5) ← BLACK (tie to GND for immediate output).

3.4 Insert TL072 into proto board

Same deal — pin 1 marked with dot/notch, top-left.

3.5 Wire TL072 power

Pin 8 (V+) ← ORANGE wire (5V). Pin 4 (V-) ← BLACK wire (GND). Note: ideally V- should be negative voltage for full swing, but 0V-to-5V output range works fine for our galvos.

3.6 Connect DAC outputs to op-amp inputs

MCP4822 VOUTA (pin8) → BROWN wire → TL072 pin 3 (non-inverting input A). MCP4822 VOUTB (pin6) → GRAY wire → TL072 pin 5 (non-inverting input B). Wire TL072 pin 1 (output A) back to pin 2 (inverting input A) — this makes it a voltage follower. Wire TL072 pin 7 (output B) back to pin 6 (inverting input B) — same thing.

3.7 Run galvo signal wires

TL072 pin 1 (output A) → BROWN wire → Galvo X driver "signal in" (analog input). TL072 pin 7 (output B) → GRAY wire → Galvo Y driver "signal in" (analog input). Also connect galvo driver GND to your BLACK ground rail.

3.8 TEST IT ✅

Power on. Use your multimeter to check: TL072 pin 8 should read ~5V. TL072 pin 4 should read 0V. MCP4822 pin 1 should read ~3.3V. If voltages are correct, the circuit is ready for firmware.

## Step 4: Laser Module

🎯 Goal: Connect the laser so the ESP32 can turn it on and off.

⚠️ LASER SAFETY: Never look directly into the beam. Never point at people or pets. Wear laser safety goggles (OD4+ for 520nm) during testing. The beam is invisible to the side — only dangerous when looking directly into it.

4.1 Wire laser power

RED wire (12V) → Laser module V+ (usually red wire from module). BLACK wire (GND) → Laser module GND (usually black wire).

4.2 Wire laser TTL control

GREEN wire from ESP32 GPIO 4 → Laser module TTL input (usually yellow or white wire from module). This lets the ESP32 turn the laser on (HIGH) and off (LOW) at high speed.

4.3 TEST IT ✅

DO NOT power on yet. First: tape the laser module to the table pointing at a wall. Put on safety goggles. THEN power on. The laser should NOT turn on yet (GPIO is LOW by default). If the laser IS on, UNPLUG immediately — your TTL wire may be loose.

## Step 5: Galvo Scanners

🎯 Goal: Mount the galvo mirrors and connect their driver boards.

5.1 Mount galvos in enclosure

The two galvo motors mount perpendicular to each other. One controls X (horizontal), one controls Y (vertical). The laser beam hits mirror 1, bounces to mirror 2, then projects outward. Most galvo sets come with a mounting bracket — use it. Align so the beam exits through a hole in the enclosure.

5.2 Connect galvo driver boards

Power: RED (12V) → driver V+. BLACK (GND) → driver GND. Signal: BROWN wire → X driver signal input. GRAY wire → Y driver signal input. The driver boards may have additional pins (±15V, etc.) — for 20Kpps sets running on 12V, you typically just need power + signal. Check your specific driver board docs.

5.3 Align the laser beam through both mirrors

This is fiddly but not hard. Power on the laser (carefully). Adjust mirror angles until the beam hits mirror 1, bounces to mirror 2, and exits the enclosure as a dot on the wall. The galvo set instructions will show the correct mirror geometry.

## Step 6: Flash Firmware & First Test

🎯 Goal: Upload code to ESP32 and see the laser draw a shape on the wall.

6.1 Install Arduino IDE

Download from arduino.cc. Add ESP32-S3 board support (Tools → Board Manager → search "esp32").

6.2 Flash the test firmware

I (Claude) will write this for you. It draws a simple square on the wall to prove everything works. Open the .ino file in Arduino IDE, select "ESP32-S3 Dev Module", click Upload.

6.3 THE MOMENT OF TRUTH ✅

Power on. Put on goggles. The laser should draw a green square on the wall. If you see a square — congratulations, you built a laser projector. If you see a line — one galvo axis isn't responding (check X or Y signal wire). If you see a dot — neither galvo is responding (check DAC wiring). If nothing — check power, check USB, re-upload firmware.

# Troubleshooting — When Things Go Wrong

Something not working? Don't panic. Here's how to debug:

"Nothing powers on" → Check USB-C cable is PD-capable (not a charge-only cable). Check PD board is set to 12V. Measure voltage at PD board output terminals with multimeter.

"ESP32 doesn't blink/respond" → Check 5V is reaching VIN pin. Try powering ESP32 via its own USB-C port (bypasses your power circuit) to verify the board itself works.

"Laser doesn't turn on when it should" → Measure 12V at laser V+ terminal. Check TTL wire is on correct ESP32 GPIO. Try connecting TTL directly to 3.3V briefly — if laser turns on, the issue is firmware/GPIO.

"Galvos don't move" → Check 12V at galvo driver. Measure DAC output with multimeter — should change when firmware runs. If DAC output is stuck at 0V, check SPI wiring (MOSI, CLK, CS pins).

"Laser draws a line instead of a square" → One galvo axis works, the other doesn't. Swap the BROWN and GRAY wires to see which axis is dead. Then check that axis's wiring: DAC → op-amp → galvo driver.

"Shape is distorted or tiny" → DAC output voltage range may not match galvo driver input range. The op-amp voltage follower outputs 0-3.3V but galvos may want ±5V. Fix: adjust the op-amp circuit to shift and scale (this is a Phase 2 refinement — a small shape that works is a huge win for Phase 1).

# You Built It! What's Next?

If you see a laser drawing shapes on your wall, you have a working prototype. That's the hardest part. Everything from here is firmware improvements:

Phase 1.5: WiFi connection: ESP32 connects to your phone's WiFi. The LayIt app sends tile coordinates. The laser draws the tile pattern instead of a test square.

Phase 2: Camera alignment: The OV5640 camera detects placed tile edges. The laser auto-corrects its projection to match the real world.

Phase 3: Bump recovery: The MPU6050 IMU detects displacement. Camera re-locks to existing tiles.

Phase 4: Enclosure & polish: Clean up wiring, mount everything in the aluminum box, add the tripod mount. Make it look like a product.

Every tile in its perfect place.

SIMCO Inc. • Patent Pending