# LayIt Laser — Hardware Build Partner Wanted

Twin Cities / Remote • Paid Project • All Components Provided

## The Project

I'm building a portable laser projector that beams tile installation patterns onto walls and floors at 1:1 scale. Think of it as a laser level on steroids — except instead of one line, it draws the entire tile grid in real time. The companion app (already built and functional on iOS) generates the layout. The laser projects it.

I have all the components, the firmware architecture designed, three provisional patents filed, and a working iOS app with 50+ commits. What I need is someone who can assemble the hardware prototype and get the basic firmware running on the ESP32.

## What You'd Be Building

A portable laser projection unit with these core components:

Laser: 200mW 520nm green laser module with TTL driver (Laserland 4060-530D-200)

Galvanometers: 20Kpps dual-axis galvo scanner set with ILDA interface — this draws the pattern

MCU: ESP32-S3-WROOM-1 (N16R8) — dual-core: Core 0 handles WiFi + camera, Core 1 runs the real-time galvo scan loop

DAC: MCP4822 dual-channel SPI DAC → TL072 op-amp buffer → galvo driver inputs

Camera: OV5640 5MP wide-angle (DVP interface) for vision-based alignment

IMU: MPU6050 6-axis accelerometer/gyro for bump detection

Power: USB-C PD trigger board (12V) + AMS1117 regulators for 5V/3.3V rails

Enclosure: 6"×4"×3" aluminum project box with tripod mount

Total BOM is ~$250-$310. I have most components already or can order the rest immediately. Full BOM with part numbers, sources, and costs is documented.

## Firmware Scope (Phase 1)

The initial firmware goal is straightforward — get the galvos drawing patterns from the app:

ESP32-S3 connects to WiFi, receives tile coordinates from the iOS app via WebSocket

Core 1 runs a tight scan loop: read coordinate buffer → write X/Y to MCP4822 DAC via SPI → toggle laser TTL

Core 0 handles WiFi, receives new pattern data, manages the coordinate buffer

Basic safety: laser auto-off after 30s of no communication, hardware interlock pin

Camera and IMU integration come in Phase 2 (vision alignment) — not needed for the first working demo

If you've done any ILDA laser project, galvo art, or laser light show work, the scan loop is familiar territory. The DAC+galvo path is standard. The ESP32 WiFi is well-documented. The novel part is the app integration, which I handle.

## What I Provide

All components (purchased or reimbursed)

Complete BOM with datasheets and wiring diagrams

Firmware architecture document (pin assignments, timing requirements, protocol spec)

The iOS app that generates and sends tile coordinates

Three provisional patents covering the system (your work is protected under NDA)

Clear scope definition — no mission creep

## What I'm Looking For

Experience with ESP32 (or similar MCU) firmware development in C/Arduino

Comfortable with SPI, I2C, and basic analog circuits (DAC → op-amp → galvo driver)

Soldering skills for through-hole and basic SMD (nothing finer than SOIC)

Bonus: experience with galvanometer scanners, ILDA, or laser projects

Bonus: 3D printing or CNC for enclosure modifications

Located in Twin Cities metro preferred, but remote works if you can ship the assembled unit back

## Compensation

This is a paid project. Budget is flexible based on experience — we can discuss hourly, flat fee, or milestone-based compensation. Ballpark: $500–$1,500 for Phase 1 (assembled hardware + basic firmware running). Phase 2 (vision alignment) is a separate scope and separate pay.

If the prototype works well and you're interested in ongoing work, there's a path to a longer engagement as we move toward manufacturing. But no pressure — Phase 1 is a clean, well-defined project.

## About LayIt

LayIt is a tile installation platform by SIMCO Inc. The app is live on iOS with 234 tile presets, AI-powered tile scanning, a visual cut tracker, and client report generation. The laser projector is the hardware companion that projects the app's layout onto real surfaces at 1:1 scale.

Three provisional patents are filed covering the laser projection system with vision-based self-correcting alignment, camera-based room measurement, and AI material geometry extraction. This is real IP with a clear path to market.

## Interested?

Email: layitapp@gmail.com

Subject line: "LayIt Laser Build" — tell me about your experience with MCUs, laser projects, or hardware prototyping.

SIMCO Inc. • Patent Pending • Confidential