LayIt Laser

Hardware Build Guide

With Camera Auto-Calibration

Version 2.0  •  January 2026

For Prototype Builders & Manufacturing Partners

# Overview

This guide provides complete instructions for building the LayIt Laser prototype—a WiFi-enabled laser projector that displays tile patterns on walls and floors. This version includes integrated camera auto-calibration for true plug-and-play operation.

Key Components:

• Green laser module (520nm, 50mW)

• Dual-axis galvanometer scanner

• ESP32-S3 microcontroller with camera interface

• OV5640 camera module for auto-calibration

• 12V power supply

• Aluminum enclosure with tripod mount

# Bill of Materials (BOM)

| Component | Specification | Est. Cost |
| --- | --- | --- |
| Laser Module | 520nm Green, 50mW, 12V | $15-25 |
| Galvanometer Set | 20K PPS, dual-axis, with mirrors | $45-65 |
| Galvo Driver Board | XY2-100 compatible, ±5V input | $20-30 |
| ESP32-S3-WROOM-1 | N16R8 (16MB flash, 8MB PSRAM) | $8-12 |
| Camera Module (NEW) | OV5640, 5MP, 160° wide angle | $30-45 |
| DAC (Dual Channel) | MCP4822 or MCP4922, SPI | $3-5 |
| Op-Amp Buffer | TL072 or similar, dual | $1-2 |
| Power Supply | 12V 3A DC adapter | $8-12 |
| Voltage Regulators | 5V (AMS1117), 3.3V (AMS1117) | $2-3 |
| Enclosure | Aluminum, 6"×4"×3", vented | $25-35 |
| Misc (PCB, connectors, etc.) | Wires, headers, screws, lens | $15-25 |
| TOTAL ESTIMATED BOM |  | $220-290 |

# System Architecture

## Block Diagram

[Phone App] ←WiFi→ [ESP32-S3] → [DAC] → [Galvo Driver] → [Galvos] → [Laser]

↑

[Camera] ← Auto-calibration feedback

## Signal Flow

1. Phone app sends pattern data via WiFi to ESP32

2. ESP32 converts vector coordinates to DAC values

3. DAC outputs 0-5V analog signals

4. Op-amp scales to ±5V for galvo driver

5. Galvo driver positions mirrors

6. Laser beam traces pattern on surface

# Camera Auto-Calibration System

This is the key differentiator that makes LayIt Laser a plug-and-play device.

## Hardware Setup

Camera: OV5640 module connected to ESP32-S3 via DVP interface

Lens: Wide-angle (160°) to capture entire projection area

Mounting: Camera aligned parallel to laser output, ~1" offset

Resolution: 640×480 sufficient for calibration (lower = faster processing)

## Calibration Algorithm

Step 1: Project 4 reference dots at known galvo positions (corners of a 12" square)

Step 2: Camera captures image with laser dots visible

Step 3: Image processing finds dot centroids (green channel threshold)

Step 4: Calculate pixel distance between dots

Step 5: Compare to expected distance, compute scale factor

Step 6: Apply correction to all subsequent projections

Total calibration time: <10 seconds

# Wiring Connections

## ESP32-S3 Pin Assignments

| GPIO | Function | Connection |
| --- | --- | --- |
| GPIO 10 | SPI CLK | DAC SCK |
| GPIO 11 | SPI MOSI | DAC SDI |
| GPIO 12 | SPI CS | DAC CS |
| GPIO 13 | Laser TTL | Laser enable (via transistor) |
| GPIO 4-9 | Camera Data D0-D7 | OV5640 parallel data |
| GPIO 18 | Camera XCLK | OV5640 clock input |
| GPIO 15/16 | Camera I2C | OV5640 SCCB (SDA/SCL) |

# Firmware Overview

The ESP32-S3 runs custom firmware with the following modules:

WiFi Manager: Handles AP mode for setup, connects to home WiFi

Web Server: Receives pattern data from LayIt app via HTTP/WebSocket

Pattern Engine: Converts tile patterns to galvo coordinates

DAC Driver: High-speed SPI output to galvo DAC

Camera Driver: OV5640 initialization, frame capture

Auto-Cal Module: Image processing, scale factor computation

# Assembly Instructions

1. Mount galvos in enclosure with proper alignment

2. Install laser module, align beam to hit galvo mirrors

3. Mount ESP32-S3 board with camera connector accessible

4. Install camera module pointing through front aperture

5. Connect DAC and galvo driver boards

6. Wire power distribution (12V → 5V → 3.3V)

7. Flash firmware via USB

8. Test and calibrate

# Safety Notes

⚠️ LASER SAFETY: 50mW green laser is Class 3R. Avoid direct eye exposure.

• Always wear laser safety glasses (OD2+ at 520nm) during testing

• Never point at people or reflective surfaces

• Include safety interlock in final product design

• Ensure proper labeling per FDA/IEC requirements

# Contact & Support

For build support, firmware updates, or manufacturing partnership:

Email: build@layitlaser.com

GitHub: github.com/layitlaser/firmware